PRODUCT BULLETIN

MODEL GI INTERNAL GEAR PUMP
[image: image1.jpg]90° port
options

SO POO
o fo 1o YoX oX oX oko;




[image: image4.jpg]- A

AMERICAN-MARSH PUMPS





The GI Internal Gear pumps operate smoothly and with equal efficiency in either direction.  They efficiently handle both thin and thick liquids and are designed for a broad range of applications at verious flows and pressures. They are also capable of operating under a suction lift condition.
Pumps are available in several materials of construction with or without relief valves.  Pumps can be supplied with conventional packing or mechanical seal. Available with 90 degree ports, which can be rotated in 45 deggree increments.

[image: image5.jpg]




MODEL GI SPECIFICATIONS

Rotor/Idler:  Heat treated cast iron pumping gears are accurately machined for quiet, efficient operation and long life.  The rotor is securely fastened to the pump shaft. Rotor end clearance can be adjusted to compensate for wear, for higher temperatures, or for higher viscosities by rotating the threaded bearing housing.  Rotor and Idler can also be constructed of bronze, steel, or stainless steel.
Bearing Surfaces:  Bearing are special wear resistant bronze.  Cast iron and carbon bearings are available.  Heavy duty sleeve bearings give positive support to pumping gears and insure long, efficient service.  Bearing grooves allow circulation of the liquid being pumped for lubrication and control of bearing temperature.  
Precision Ground Shafts:   The steel shaft is induction hardened in the bearing and packing areas and are precision ground to exacting standards for maximum life.  Hardened stainless steel shafts are available.
Pump Housing:  All castings are of high grade cast iron.  Precise manufacturing tolerances provide minimum clearances for maximum pumping efficiency.  Double row ball or tapered bearing are for axial thrustt control.  Large diameter threaded bearing housing allows for easy reamoval of seals.
Stuffing Box/Seal Chamber:  The stuffing box is sized for a standard packing, a variety of component seals and cartridge type mechanical and lip seals.  
MODEL: ALL GI PACKED PUMPS

[image: image2.jpg]



	Item Number
	Item Description
	Num. Req.
	
	Item Number
	Item Description
	Num. Req.

	1
	Locknut
	1
	
	17
	Bracket and Bushing Assembly
	1

	2
	Lockwasher
	1
	
	18
	Bracket Bushing
	1

	3
	End Cap
	1
	
	19
	Bracket Gasket
	1

	4
	Lip Seal
	1
	
	20
	Pipe Plug
	1

	5
	Bearing Spacer Collar (Outer)
	1
	
	21
	Casing (Tapped or Flanged)
	1

	6
	Ball Bearing
	1
	
	22
	Rotor and Shaft Assembly
	1

	7
	Bearing Spacer Collar (Inner)
	1
	
	23
	Idler and Bushing Assembly
	1

	8
	Ring, Half Round (Not on GI-H & GI-HL)
	1
	
	24
	Idler Bushing
	1

	9
	Bearing Housing
	1
	
	25
	Head Gasket
	1

	10
	Packing Gland
	1
	
	26
	Idler Pin
	1

	11
	Packing Gland Nut
	2
	
	27
	Head and Idler Pin Assembly
	1

	12
	Packing Gland Capscrew
	2
	
	28
	Capscrew for Head
	6

	13
	Packing 
	1
	
	29
	Relief Valve Gasket
	2

	14
	Packing Retainer Washer
	1
	
	30
	Internal Relief Valve
	1

	15
	Capscrew for Bracket
	8
	
	31
	Capscrew for Valve
	8

	16
	Grease Fitting
	1
	
	
	
	


MODEL: ALL GI COMPONENT MECHANICAL SEAL PUMPS

[image: image3.jpg]



	Item Number
	Item Description
	Num. Req.
	
	Item Number
	Item Description
	Num. Req.

	1
	Locknut
	1
	
	15
	Mechanical Seal
	1

	2
	Lockwasher
	1
	
	16
	Bracket Gasket
	1

	3
	End Cap
	1
	
	17
	Pipe Plug
	1

	4
	Lip Seal
	1
	
	18
	Casing (Tapped or Flanged)
	1

	5
	Bearing Spacer Collar (Outer)
	1
	
	19
	Rotor and Shaft Assembly
	1

	6
	Ball Bearing
	1
	
	20
	Idler and Bushing Assembly
	1

	7
	Bearing Spacer Collar (Inner)
	1
	
	21
	Idler Bushing
	1

	8
	Ring, Half Round (Not on GI-H & GI-HL)
	1
	
	22
	Head Gasket
	1

	9
	Bearing Housing
	1
	
	23
	Idler Pin
	1

	10
	Capscrew for Bracket
	8
	
	24
	Head and Idler Pin Assembly
	1

	11
	Lip Seal
	1
	
	25
	Capscrew for Head
	6

	12
	Grease Fitting
	1
	
	26
	Relief Valve Gasket
	2

	13
	Bracket and Bushing Assembly
	1
	
	27
	Internal Relief Valve
	1

	14
	Bracket Bushing
	1
	
	28
	Capscrew for Relief Valve
	8


PRODUCT BULLETIN			INDUSTRIAL


MODEL GI INTERNAL GEAR	Crude Oil, Acids, Gasoline, Naphtha,  Petrochemical, Organic Fluids, Cargo Transfer, Asphalt, Molasses, Roofing Compounds, Printing Inks, Fuel Oil























Bulletin 200GI


Edition 1








Bulletin 200GI


Edition 1









































1
1

