PRODUCT BULLETIN

MODEL REC-SJ END SUCTION
[image: image1.jpg]


[image: image2.jpg]_.',\ S ] Bk
VGl ] “.\F'J
|
alee r“ 3/3c8¢
v6v TS
m % \ \_/ \_/ \_/ </ U U Ul
I mw L QN
\ (Ll
NE=
| |[BVe=
= VT
AO0S» | (=
POOGx | (D= — =
W .|
9005 | m g
|
MO0S —

: _ ““mw 3008
MOOS i ®
X00Gx \ A

#
. 001G+
g01Gx ;
VOIS« N
400G+ i
B— 5C8Cx
Yooy
000G+ m—‘ \\\\\ 8005+
i\ A
YOOGS+




[image: image3.jpg]- A

AMERICAN-MARSH PUMPS





American-Marsh Model REC-SJ End Suction pumps are used in a variety of industries.  These efficient small centrifugal pumps are designed for long life and low operational costs.  Each model is of the back pull out design so when the need arises for repair or inspection, system piping does not need to be disturbed.  REC-SJ models offer tangential discharge and can be mounted in several different orientations.  The REC-SJ product line surpasses other competitor product lines in that it is available in sizes 1x1.25-5 through 1.25x1.25-5.

Material Specifications

	PART
	CONSTRUCTION

	
	BRONZE FITTED

	Casing
	Cast Iron

	Impeller
	Bronze

	Shaft
	Stainless Steel

	Case Wear Ring
	Bronze

	Bolts, Studs & Nuts
	Steel

	Mechanical Seal
	Carbon/Ceramic/Buna

	Voltage/Phase
	230 Volt/Single


MODEL REC-SJ SPECIFICATIONS

Casing:  The casing is constructed of high tensile cast iron or other specified material.   It is of the volute type, carefully and accurately proportioned to permit smooth flow and to convert high velocity energy of the fluid as it leaves the impeller into pressure.  Suction and discharge nozzles are casted integral with the volute and are NPT female.  All REC-SJ models feature a 175 psi case working pressure.  The casing has a cast integral tangential discharge ports.  Necessary vent and drain openings are provided.

Impeller:  The impeller is of the end suction type, casted in one piece of bronze.  All impellers are hydraulically and dynamically balanced prior to assembly and are available in four (4) trims designed to maximumize available horsepower.  

Case Wear Ring:   Standard enclosed impellers are designed with integral case wear rings accurately turned to provide close running fits in casing.  The diameters of these rings are such as to reduce end thrust to a minimum.
Casing Adapter:  The casing adapter is constructed of cast iron and houses the mechanical seal. 

Seal Chamber:  The seal chamber is sized for a standard, front loading mechanical seal.  The mechanical seal is constructed of a carbon rotary face, a ceramic stationary face and buna elastomers standard.

Motor:  All REC-SJ models utilize American-Marsh, 56Y, square flange motors.  Motors are available in TEFC enclosures, single phase and horsepowers from 1/2 through 1-1/2 HP.

REC-SJ Sectional Drawing

	Item Number
	Item Description
	Num. Req.
	
	Item Number
	Item Description
	Num. Req.

	1A
	Casing
	1
	
	*500E
	Motor Outboard Bearing
	1

	2C
	Rubber Foot Mount
	1
	
	*500F
	Motor Terminal Board
	1

	2D
	Pump Foot
	1
	
	*500G
	Motor Outer Frame
	1

	11A
	Impeller
	1
	
	*500J
	Motor Stator Windings
	1

	15A
	Case Wear Ring
	1
	
	*500K
	Motor Tie Rod
	4

	*41A
	Shaft & Rotor Assembly
	1
	
	*500L
	Motor Inboard Cap
	1

	331B
	Mechanical Seal
	1
	
	*500M
	Motor Inboard Bearing
	1

	351A
	Casing Gasket
	1
	
	*500W
	Motor Outboard Cap
	1

	383E/F
	Casing Stud & Nut
	4
	
	*500X
	Motor Hex Nut
	1

	411B
	Plug, Drain
	1
	
	*500Y
	Motor Inner Frame
	1

	494
	Shaft Deflector
	1
	
	*510A
	Motor Capacitor Clamp
	1

	*500A
	Motor Cooling Fan Split Pin
	1
	
	*510B
	Motor Capacitor
	1

	*500B
	Motor Cooling Fan Cover
	1
	
	*510C
	Motor Wiring Gland
	1

	*500C
	Motor Cooling Fan
	1
	
	
	
	


*American-Marsh motor item.

Recommended spare parts are in BOLD.
PRODUCT BULLETIN			MUNICIPAL				COMMERCIAL				INDUSTRIAL


MODEL REC-SJ END SUCTION		Quench Water, Filtration, Transfer,	HVAC, Booster, Condensate Return,	Process, Transfer


						Filter Backwash, Circulation		Supply Water














Bulletin 300REC-SJ


Edition 1

















Bulletin 300REC-SJ


Edition 1














1

